Task

Une entreprise en pleine croissance comptant plus de 2 millions de clients dans le monde. L’entreprise s’occupe de la vente et de l’achat de produits alimentaires, activités principalement gérées depuis le siège. Elle prévoit d’ouvrir une succursale. Ainsi, l’entreprise recherche de jeunes diplômés en informatique pour concevoir le réseau de cette succursale.

Le réseau doit fonctionner indépendamment de celui du siège.

Étant un petit réseau, l’entreprise a les exigences suivantes lors de sa mise en œuvre :

a) Un routeur et un commutateur doivent être utilisés (tous deux des produits CISCO).

b) 3 départements (Administration/IT, Finance/RH et Service client/Réception).

c) Chaque département doit être dans un VLAN différent.

d) Chaque département doit disposer d’un réseau sans fil pour les utilisateurs.

e) Les hôtes du réseau doivent obtenir automatiquement une adresse IPv4.

f) Les appareils de tous les départements doivent pouvoir communiquer entre eux.

Supposez que le fournisseur d'accès Internet (ISP) ait attribué un réseau de base : 192.168.1.0.

Vous, en tant que jeune ingénieur réseau nouvellement embauché, devez concevoir et mettre en œuvre un réseau en tenant compte des exigences ci-dessus.

Subnetting

Base network: 192.168.1.0

No of subnets = 3 (IT, Finance, Reception departments)

2^n >= 3 ==== n=2

Class C = 255.255.255.0 === 11111111. 11111111. 11111111.00000000

After borrowing 2 bits

11111111. 11111111. 11111111.11000000

New Subnet Mask :255.255.255.192

1 Subnet(Vlan 10)

Network IP :192.168.1.0

Broadcast IP :192.168.1.63

Host Range: 192.168.1.1192.168.1.62

2 Subnet(Vlan 20)

Network IP :192.168.1.64

Broadcast IP :192.168.1.127

Host Range: 192.168.1.65192.168.1.126

3 Subnet(Vlan 30)

Network IP :192.168.1.128

Broadcast IP :192.168.1.191

Host Range: 192.168.1.129192.168.1.190

Subinterfaces configuration in router

Int g0/0/0.10

Encapsulation dot1Q 10

Ip add 192.168.1.1 255.255.255.192

Int g0/0/0.20

Encapsulation dot1Q 20

Ip add 192.168.1.65 255.255.255.192

Int g0/0/0.30

Encapsulation dot1Q 30

Ip add 192.168.1.129 255.255.255.192

WIFI Configuration

IT department

Access point configuration on port 1

SSID :ADMIN-WIFI

Password :Admin@123

Security : WPA2-PSK

Finance/HR department

Access point configuration on port 1

SSID :FINANCE-WIFI

Password :Finance@123

Security : WPA2-PSK

Reception department

Access point configuration on port 1

SSID :RECEPTION-WIFI

Password :Reception@123

Security : WPA2-PSK

Dhcp configuration in router

It department conf

Ip dhcp pool Admin-Pool

Net 192.168.1.0 255.255.255.192

Default-router 192.168.1.1(ip given in subinterface)

Dns-server 192.168.1.1 (ip given in subinterface)

Domain-name Admin.com

Finance department conf

Ip dhcp pool Finance-Pool

Net 192.168.1.64 255.255.255.192

Default-router 192.168.1.65(ip given in subinterface)

Dns-server 192.168.1.65 (ip given in subinterface)

Domain-name Finance.com

Reception department conf

Ip dhcp pool Reception-Pool

Net 192.168.1.128 255.255.255.192

Default-router 192.168.1.129(ip given in subinterface)

Dns-server 192.168.1.129 (ip given in subinterface)

Domain-name Reception.com

[-Configure all Pc’s and Printers to request Ip address by changing configuration to dhcp

-Connect laptops and smartphones to AP (they will have ip add automatically)

Add WPC300N (Adaptateur réseau sans fil) in laptop to connect to wifi]